Yes, you can get resolution of 0.1 or even 0.01 degrees if you use oversampling technique (google "oversampling"). There are a lot of applications and examples, practically you can get any resolution you want if you apply correctly the oversampling conditions:
1. Limit the spectar of the input signal (some low pass filter, reject all HF)
2. Apply correctly noise to the signals (some triangular noise is the best)
3. Collect enough samples. The formula is Number_samples = 4 ^ n, where n is the number of the additional bits you want to get. For example you want to squeeze from your 10 bit ADC 14 bits, then make 256 samples (and follow steps 1 and 2) and you'll finish with 14 bits. 

I made some projects with oversampling and I can ensure you that the theory really works, you get pure and clear additional bits (I went up to 16 bits from 10 bit ADC). So instead of going to buy an expensive 16 bit ADC just spent a little more time searching the net and apply the oversampling.

Etat de l’art de la mesure de température :

Mesure par thermocouple

Mesure par RTD (pt100)

Mesure par fibre de bragg

Besoin de la MECA:

Temperature ambilante de la mesure :-30°C

Précision :environ 0.1°C

Résolution : 0.01°C

Nombre de voies de mesures :20 environs

Fréquence d’echantillonage :1s

Besoin portant sur un système de mesure complet (capteur>DAQ>PC)

Commande de plusieurs resistance chauffante :10 à 15 voie pour Ptot de 150W

Alimentation HAMEG HMP 4030 	=>1488

Interface ETH				=>228

Ce qui nous fait un cout du système de mesure variant de 7216 à 8346 hors capteurs.

PT100 10ppm

Capteur fini PT100 adhésif : 20*27.7=554€

Pate thermique :15.50

Element PT100 :20*6.3=126€

Cable=55.4€